## Comparison Chart of Vaginal Laser Types and Clinical Settings

| Laser Type | Wavelength | Pulse Mode | Typical Energy / Fluence | Spot Size | Recommended Sessions | Session Interval | Common Indications | Typical Downtime | Key Safety Precautions | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO₂ (Fractional) | 10,600 nm | Fractional/ Pixel | 30-40W, 350–400 J/cm² | 7-10 mm | 3–6 | 4–6 weeks | GSM, laxity, SUI, scarring, aesthetics | 1–3 days | Eye protection, cooling | Most studied; gentle ablation/ remodeling |
| Er:YAG | 2940 nm | SMOOTH, MSP | 3.25–6.0 J/cm² | 7 mm | 2–3 | 4–6 weeks | GSM, mild prolapse, laxity, aesthetics | <3 days | Visual tissue monitoring | Non-ablative; minimal downtime, precise |
| Diode | 1470 nm | Fractional | Device-dependent | Variable | 3 | 4 weeks | GSM, mild atrophy, sexual dysfunction | <3 days | Manufacturer protocols | Mild thermal, very safe; growing evidence |
| Pixel/ Fractional | Variable | Pixel/ Fractional | 50mJ/pixel, 5% coverage | Variable | 3–5 | 4 weeks | All indications, aesthetics | 1–2 days | Avoid overlapping pulses | Uniform coverage; customizable settings |