# Transcutaneous RF Procedural Guidelines

## 1. Pre-Procedure Checklist

To be evaluated by the prescribing physician
Medical records, signed patient consent, and test results.

- Review Patient History & Signed Consent

  - Negative Pap smear (document date & result)

  - Pelvic exam within last 12 months (attach report)

  - Negative pregnancy test (attach lab result)

  - Negative for infections/wounds (HSV, lesions, UTI, yeast) — clinical examination notes

- Patient Photographs (as per practice policy)

  - Evidence: Stored and labeled before procedure

- Hair Removal for External Procedures

  - Shaving: At least 1 day prior

  - Waxing/laser: 5–7 days prior

  - Evidence: Patient self-report or clinic record

## 2. Procedure Set-Up

Equipment log and visual inspection checklist

- System Preparation

  - Ensure RF generator powered, positioned, switched on

- Grounding Pad

  - Correct attachment per Instructions for Use

- Extension Cable & Electrode Connections

  - Correct orientation

  - RFDE-VA electrode attached without twisting

## 3. Pre-Procedure Preparation

Nursing notes & patient verbal confirmation

- Patient empties bladder

- Patient positioned in lithotomy position

- Water-based ultrasonic gel applied (moderate coating, warmed if possible, no glycerin/colored gel)

- Generator Set-Up

  - Start at 35°C, increase to 42–47°C as tolerated (max 47°C)

  - Therapeutic target: 42°C

## 4. Procedure Execution

Procedure record including temperatures, timings, and areas treated

### A. Vaginal Canal

- Optional cleaning prior

- Start at 45–47°C

- Insert electrode until resistance, withdraw 2–3 cm (max depth 7 cm)

- Treat 2–7 cm zone

- Duration: 12–20 min internally

### B. Labia Majora / Labia Minora

- Start at 42°C, increase up to 47°C per tolerance

- Treat unilaterally unless area is small

- Duration: 12–20 min externally

Operational Notes

- Electrode in full skin contact, moderate pressure

- Foot pedal initiates RF emission (solid blue light, beep every 5 sec)

## 5. Clinical Pearls

Practitioner observation notes & patient feedback forms

- Keep electrode moving

- Maintain full tissue contact with active tip

- Use varying motion paths (withdraw/advance, sweeping, half-moon)

- Maintain ACT temperature near SET temperature

- Adjust temp per patient feedback & tissue erythema

- Avoid cervix contact & urethra area

- Only treat up to 1 cm above clitoris

## 6. Post-Procedure Care

Discharge notes & patient education record

- Cleanse skin to remove gel/cream

- Mild erythema or swelling may occur — usually resolves in ~30 min

- Provide post-care instructions, including when to contact physician

- Schedule next session